iDSC1816

...

The iDSC 1816 comes with DSCview included, and software support for DASYLab, HP VEE, LabVIEW, LabWindows/CVI, and MATLAB.  With DSCview software on your PC, you can customize the filter characteristics for each channel, and download them to the iDSC board.  Download DSCview and check it out.

The iDSC 1816 also includes programming support for Visual Basic, Visual C++, and other Windows 95/98/NT/2000 programming environments through the DSCIO.DLL, a standard Windows DLL interface.  In addition, development environments like Delphi and C++ Builder are supported through a native DSC Component.

DSCview

DSCview, a complete Windows application, allows immediate and easy access to the iDSC1816.  DSCview requires no programming, and provides a graphical interface to:

save and load workspaces

select system options

design and configure filters

perform signal conditioning

configure multiple iDSC boards

output data in a graph and table

disk log data to a text or binary file

server disk log data to a binary file by the server

Microstar Laboratories has included appropriate software from DSCview in its support for other interfaces that, like DSCview, require no programming: DASYLab, HP VEE, LabVIEW, LabWindows/CVI, and MATLAB.  We also have included the same user functionality for Visual Basic, Visual C++, and other Windows 95/98/NT/2000 programming environments through the DSCIO.DLL, and Delphi and C++ Builder development environments through a native DSC Component, for customers to program their own user interfaces.  All user interfaces share a common look and fell to designing and configuring filters.

iDSC Filters

iDSC filters are activated through the Configuration Window, which has a Filter Design tab to display the Filter Design Screen and an Input tab to display the Input Screen.

Filter Design Screen

When the Filter Design tab is selected, it activates the Filter Design Screen.  There are eight tabs that the user can name to identify up to eight different filter designs: a different filter design for each channel, if required. Selecting a tab puts that screen on top in the window.

The following figure displays the Filter Design Screen.

<show picture>

The user can enter the following parameters.

Filter Name - a unique name for each filter design

Filter Type - the filter type, lowpass or bandpass

Sharpness - the number of taps in the final filter stage, an odd number in the

range 37 to 255 (range dependent on sample rate) that determines the

sharpness of the corner frequency response

Low Cutoff Frequency - the first transition band, expressed in Hertz, in the

range 2% to 80% of the Nyquist frequency

Low Cutoff Slope - the width of the first transition band, expressed as a %

of the Nyquist frequency, in the range 0 to 80

High Cutoff Frequency - the second transition band, expressed in Hertz, in

the range 2% to 80% of the Nyquist frequency

High Cutoff Slope - the width of the second transition band, expressed as a %

of the Nyquist frequency, in the range 0 to 80

Attenuation - the response in the stop band, expressed as a value, in the

range 6.0 to 12.0

In addition, the user can access these additional parameters through the right mouse button.

Crosshair Track - tracks the filter response curve to determine the exact

cutoff frequency, cutoff slope, or attenuation at a particular point.

Y Display - displays the filter's amplitude response in linear, linear zoom,

log, log zoom, or unit step form

Defaults Load - loads the default filter response graphs for a particular sample rate

Copy - copies the filter design parameters from the filter design to the clipboard

Paste - pastes the copied filter design parameters from the clipboard to

another filter design

Input Screen

When the Input tab is selected, it activates the Input Screen.

The following figure displays the Input Screen.

<show picture>

The user can enter the following parameters.

Sample Rate - the sample rate, ranging from 8 s/s/ch to 153,600 s/s/ch

Input Range - the input range, +/- 5 V or +/- 10 V

Filter Name - maps an input pin to a named filter design

Enabled - enables or disables each input pin

Iin addition, the user can view the following read only properties.

Channel Count - the number of enabled input pins

Group Delay - the amount of time it takes for a reading to pass through the

digital filters, expressed in units of seconds

Input Pin - eight input pins, A0 through A7

Cutoff Frequency - the first and second (if exists) transition band, expressed

in Hertz

Signal Conditioning

Signal conditioning is activated through the External Board tab, which is hidden by default and must be explicitly enabled.  The External Board Screen topic describes how to enable this tab.

External Board Screen

When the External Board tab is selected, it activates the External Board Screen.  To enable the External Board tab, the user must first select System|Board Setup Display from the DSCview menu to activate multiple iDSC boards support.  After selecting the iDSC board of interest, the user must use the right mouse button to select External Board Enable.

The following figure displays the External Board Screen.

<show picture>

The user can enter the following parameters.

Input Type - the type of input signal, DC coupling, AC coupling or Excitation

Input Range - the input range, +/- 10 mV, +/- 20 mV, +/- 50 mV, +/- 100 mV,

+/- 200 mV, +/- 500 mV, +/- 1 V, +/- 2 V, +/- 5 V, or +/- 10 V

Input Offset  - the input offset, a voltage in the range of the Input Offset Range

Output Excitation - the output excitation, 0 V, 1 V, 2 V, 5 V, or 10 V

In addition, the user can view the following read only properties.

Input Pin - eight input pins, A0 through A7

Input Offset Range - the input offset range, +/- 0.5 V, +/- 1.0 V, +/- 2.5 V, or

+/- 5.0 V (supported range dependent on input range)

Multiple iDSC Boards

Multiple iDSC boards support is activated through the Group Interface, which is selected through the DSCview menu, System|Board Setup Display.

Group Interface

When the DSCview menu, System|Board Setup Display is selected, it activates the Group Interface.

The following figure displays the Group Interface.

<show picture>

The user can enter the following parameters.

DSCs - the number of iDSC boards in the system

Address - the unique iDSC address

Mode - the mode of the iDSC board, Independent, Slave of Xxx (where Xxx is

the name of the iDSC board) or Master (automatically set when the slaves

are selected)

In addition, the user can access these additional parameters through the right mouse button.

External Board Calibrate - calibrates the external board if the external board

is enabled

External Board Enable - enables the external board

Raw Data - selects raw data rather than filtered data from the iDSC

Remote Master - configures a Master iDSC board as a remote master

Server Disk Log - configures the server disk log parameters which includes

the filename, flags, file share mode, open flags, and file flags attributes

<show picture>

Copy - copies the entire iDSC configuration from the iDSC to the clipboard

Paste - pastes the copied iDSC configuration from the clipboard to another iDSC

Output Data Screens

There are four types of output data screens: graph, table, disk log, and server disk log.  The user may have any number of graph, table and disk log screens.  However, there is only one server disk log screen and it emerges automatically if server disk logging is enabled.

Graph Screen

...

The following figure displays the Graph Screen.

<show picture>

Table Screen

...

The following figure displays the Table Screen.

<show picture>

Disk Log Screen

...

The following figure displays the Disk Log Screen.

<show picture>

Server Disk Log Screen

...

The following figure displays the Server Disk Log Screen.

<show picture>

Delphi/C++ Builder iDSC Support

Delphi and C++ Builder development environments support the DSC Component (DSCC) Interface.  The DSCC provides a link between these development environments and the iDSC board.  It is a native component that compiles directly into the executable.  These environments also support the DSCIO.DLL Interface.

The DSCC interface supports Delphi 3, Delphi 4, Delphi 5, C++ Builder 3, C++ Builder 4, and C++ Builder 5 development environments. When the DSC Component Library is installed, the TDsc and TDscGroup objects appear on the Microstar tab with the label DSC and DSCs.

The DSCC interface within the Delphi 5 development environment is shown below.

<show picture>

In the Delphi and C++ Builder development environments, the input configuration and filter designs are saved in the user's form, allowing for full design time configuration of a filtering application.  In addition, all of the design time configuration can be manipulated during run time.

The TDsc object from the Delphi 5 development environment is shown below.

<show picture>

When the ConfigDialog property button of TDsc is depressed, the iDSC filters configuration dialog pops up.  The iDSC filters configuration dialog is shown below with the Filter Design tab selected.

<show picture>

The TDscGroup object from the Delphi 5 development environment is shown below.

<show picture>

When the ConfigDialog property button of TDscGroup is depressed, the multiple iDSC boards configuration dialog pops up.  The multiple iDSC boards configuration dialog is shown below with the Input tab selected.

<show picture>

Visual Basic/C++ iDSC Support

Visual Basic, Visual C++, and other Windows 95/98/NT/2000 programming environments support the DSCIO.DLL Interface.  The DSCIO.DLL provides a link between these programming environments and the iDSC board.  It allows the user to easily program a custom user interface in any of his or her preferred programming environments.